Changing the Ink Refill of a Columbus Ballpoint Pen
www.OhioPenworks.com


The Columbus Ballpoint Pen accepts a Parker style refill.


Unscrew the front portion of the pen from the pen barrel.


Remove the old ink refill from the barrel. DO NOT LOSE THE SPRING!


Remove the spring from the old refill and place it over the tip of the new refill.  Sometimes pen springs have different diameter ends.  If this is the case the spring should be oriented with the smallest opening at the top and the largest opening at the bottom of the refill.
Insert the new refill into the barrel.  DO NOT FORCE THE REFILL!  When the top of the refill is aligned with the proper opening in the pen's transmission it will easily slide into place. This may require that you rotate the refill slightly by rolling it with your fingers.  Forcing the refill can cause the pen to not operate properly.
Replace the front end of the pen by screwing it into the barrel.  Check for proper operation of the pen.
Ohio Penworks recommends a high quality ink, such as Schmidt or Private Reserve Easy Flow 9000 for your pen. 
+
